Recettage : Remplacement des points d’accès sans fil de l’EHPAD de Laignes

# Recettage

L’EHPAD de Laignes devait se munir d’un réseau Wi-Fi plus performant, tout en pouvant être interopérable avec le Centre Hospitalier de la Haute Côte-D’Or. Pour cela, il a fallu remplacer les points d’accès sans fil déjà présent sur le site par du matériel fourni par le CHU de Dijon.

Au sein de l’EHPAD de Laignes, il y a désormais 12 bornes AVAYA qui diffusent un seul SSID (avec mot de passe). Ces bornes ont été configurées avec une configuration « de base » qui a servi à chacune des bornes après quelques modifications. Il semble important de spécifier que les bornes ont étés fixées soit aux murs, soit aux plafonds. Les bornes ont été rentrées dans le GLPI du CH de Semur-En-Auxois, ainsi que leurs adresses IP et les adresses MAC des port Gigabit1-PoE.

Les bornes ont pour base commune « RES-LAIXXX-[Numéro de série] », la liste des bornes est la suivante, tout d’abord le numéro de série, puis le « hostname » de la borne :

A381714053688 - RES-LAI001-A381714053688

A381712053462 - RES-LAI002- A381712053462

A381714053620 - RES-LAI003-A381714053620

A3817140535D6 - RES-LAI004-A3817140535D6

A3817140536EE - RES-LAI005-A3817140536EE

A38171405364C - RES-LAI006-A38171405364C

A381712053586 - RES-LAI007-A381712053586

A3817120534FC– RES-LAI012-A3817120534FC

A381712053414 - RES-LAI009-A381712053414

A3817140535F6 - RES-LAI010-A3817140535F6

A38171405361A - RES-LAI011-A38171405361A

A38171205354A - RES-LAI013-A38171205354A

Il y a trois zones blanches non résolues pour l’instant, un dans l’aile USA de l’EHPAD, un dans la salle d’animation, et un avant le salon, mais après l’intersection qu’on rencontre vers le répartiteur à droite. La zone blanche à cette intersection a été corrigé en y mettant une borne de plus. Concernant les autres, le service technique du CH de Semur-En-Auxois sera sollicité pour tirer des prises, pour d’autres bornes. Le plan de l’EHPAD est ci-dessous.

Le SSID diffusé par les bornes a pour nom « Laignes-hopital2 », et une sécurité en WPA2 PSK.

Le SSID diffusé pour les visiteurs a pour nom « Laignes-Visiteur » et une sécurité en WPA2 PSK.

Concernant l’accès aux bornes via navigateur, on peut accéder à l’interface de management Web avec les identifiants par défaut. Les bornes sont configurées en DHCP.

Concernant le paramétrage des switches et du DHCP au sein du réseau, chaque SSID possède son VLAN configuré sur chacun des 3 switches de Laignes. Les baux DHCP ont été configurés sur une période de 8 jours. Les adresses IP des bornes ont étés fixées sur le serveur DHCP, de façon à ne pas perdre l’adressage si le switch redémarre, par exemple.

Deux étendues DHCP ont été créés, avec leurs plages d’adresses IP, leurs passerelles, leurs masques de sous-réseau et les VLANs en qtagged sur les bons ports. Il a fallu créer deux étendues DHCP pour chacun des SSIDs, avec des plages, l’IP des serveurs DNS, le nom de l’étendue, la passerelle, un masque de sous-réseau ainsi qu’un éventuel serveur WINS (pas présent ici).

# Annexe 1 – Configuration « de base » des bornes

!

configure

!

description

hostname RES-LAI001-A381714053688

location "NIVEAU1_ACC"

exit

!

system-info

! hardware-configuration

! ======================

!  model: WAP9144, 1.0GB (1.2GHz)

!

!  component      part number       serial number     date

!  ---------      ---------------   ---------------   -----------------

!  system         WAP9144           A381714053688     2017-Apr-10  2:29

!  controller     100-0183-001      0000341640        2017-Apr-10  2:29

!  iap module 1   100-0182-001.A    0001001853        2017-Apr-10 10:34

!  iap module 2   100-0182-001.A    0001001801        2017-Apr-10 10:34

!

!  interface    mac address(es)

!  ----------   --------------------------

!  iaps         64:a7:dd:7c:08:80-7c:08:9f

!  gigabit  1   64:a7:dd:05:36:88

!  gigabit  2   64:a7:dd:05:36:89

!

! software-configuration

! ======================

!  component         version

!  ---------------   -----------------------------------

!  scd firmware      5.00  (Oct  1 2012), Build: 4651

!  boot loader       7.1.0 (Sep 06 2017), Build: 7156

!  iap driver        3.1.0 (Dec 15 2017), Build: 2203

!  system software   8.4.4 (Dec 21 2017), Build: 7313

!  app signatures    unknown

!  license key       1A7ML-KBT9C-GMMLF-MFQEY

!  license features  AOS 8.4 for 2 4x4 radios + RF Performance Manager + RF Analysis Manager + RF Security Manager + 802.11ac + 802.11n

exit

!

boot-env  ! (local boot images)

verify bootfile_active XS-8.4.4-7313.bin

verify bootfile_backup X*.bin

exit

!

administrator

reset

edit admin password enc "$1$30c0e241$LVn1syCzqY0L9Vjkx6pnI0" privilege read-write

exit

!

vlan

reset

!

add  "DATA_WIFI_LAI"  number  73

fabric-attach off

fast-roam     on

exit

!

add  "DATA_WIFI_PUBLIC_LA"  number  76

fabric-attach off

fast-roam     on

exit

exit

!

interface gig1

autoneg      on

mtu          1500

ipv4 dhcp

ipv6 dhcp    off

ipv6 addr

management   on

led          on

up

exit

!

interface gig2

led          on

up

exit

!

dns

domain

server1  10.80.100.1

server2

server3

use-dhcp on

exit

!

interface iap  ! (global settings)

!

global-settings

all-down

rf-monitor		timeshare

country-code        FR

dot11h-support	on

max-stations	unlimited

!

intrude-detect

!

rogue-ap

reset

add "00:1b:4f:*" known    bssid

add "2c:f4:c5:*" known    bssid

add "58:16:26:*" known    bssid

add "5c:e2:86:*" known    bssid

add "64:a7:dd:*" known    bssid

add "70:38:ee:*" known    bssid

add "70:52:c5:*" known    bssid

add "b0:ad:aa:*" known    bssid

add "cc:f9:54:*" known    bssid

add "f8:15:47:*" known    bssid

exit

exit

exit

!

global-a-settings

rates basic      6   12    24

rates supported  6 9 12 18 24 36 48 54

max-stations     unlimited

exit

!

global-n-settings

rates basic       none

rates supported   all

exit

!

global-bg-settings

rates basic      6   12   24

rates supported  6 9 12 18 24 36 48 54

max-stations     unlimited

exit

exit

!

management

avcon     on

!

spanning-tree     off

!

banner pre-login  none

banner post-login none

exit

!

date-time

timezone      1 0

dst-adjust    on

ntp primary   10.80.100.1

ntp primary   auth    none

ntp primary   key-id  1

ntp primary   key enc

ntp           on

exit

!

snmp

!

v2

on

exit

!

v3

off

exit

exit

!

security wpa  ! (global settings)

eap         on

psk         off

aes         on

tkip        off

rekey       never

exit

!

security wep  ! (global settings)

key 1  size not-set

key 2  size not-set

key 3  size not-set

key 4  size not-set

default-key 1

exit

!

ssid

reset

edit "avaya" disable

!

add  "Laignes-Visiteur"

band        both

broadcast   on

vlan        76

vlan pool   none

qos         0

traffic     limit-sta  kbps unlimited

encryption  none global-settings

auth        open

disable

exit

!

add  "Laignes-hopital2"

band        both

broadcast   on

vlan        73

vlan pool   none

qos         0

encryption  wpa2 unique-settings

auth        802-1x

passphrase  [mot de passe caché]

psk         on

eap         off

enable

exit

!

del  "avaya"

exit

!

ssid   ! (iap mappings)

edit "Laignes-Visiteur" active-iaps all

edit "Laignes-hopital2" active-iaps all

exit

!

filter

reset

stateful            enable

enable

exit

!

interface iap1

band          2.4GHz

!antenna       internal omni

channel       11

channel       monitor timeshare

bond-40Mhz    off

bond-80Mhz    off

mode          gn

cellsize      max

rx-threshold -90

tx-power      20

up

exit

!

interface iap2

band          5GHz

!antenna       internal omni

channel       56

channel       unlocked

bond-40Mhz    on

bond-80Mhz    off

mode          anac

cellsize      max

rx-threshold -86

tx-power      15

up

exit

!

interface iap

!

wds

reset

allow-stations   off

exit

global-settings all-up

exit

!

station-assurance

enable

exit

!

roaming-assist

enable

threshold -15

data-rate 6

devices   notebook phone tablet player

exit

!

lldp

on

exit

!

end

!

Il semble important de noter que cette configuration ne correspond pas à la configuration finale des bornes. La configuration ici présentée est la base réemployée sur chacune des bornes lorsqu’elles ont été déployées sur Laignes.

# Annexe 2 – Diagramme de Gantt

Il est important de noter que ce diagramme de Gantt ne correspond pas à l’ordre réel des choses, celui-ci est le dernier diagramme fait, datant du 4 Juin.

Ce diagramme a été réalisé le Lundi 1er Juillet, l’après-midi.